Before a set of  candidate network routes can be aggregated, they must be in the _______________, and the routes in binary format must have the _____________.

Describe each of the following VLAN membership configuration methods:

(a) Dynamic VLAN

(b)   Static VLAN.

Ans:

(a) Static Vlans - In this method a specific switch port is assigned to a VLAN. This means that any device connected to that switch port will become a member of the VLAN configured on that port. If a Host machine is moved to a different location ( and the user wants membership of the original VLAN ) then the administrator has to manually configure that port to be a member of the desired VLAN.

(b) Dynamic Vlans - In this scenario , there is a VLAN database which has the information regarding which MAC address belongs to which VLAN. So when a user Connects to a network , the database is accesses and the host machine becomes a member of the VLAN ( as configured in the database ). The main advantage is that a host machine can be physically moved to a different location but its VLAN membership will remain the same.

(b) Now a user with a portable computer wants to be able to plug into either switch and remain in VLAN-3 without any manual configuration. How should you configure one of the switch ports in each switch to facilitate this?

ans:    the logical design is same as (a), but additional ports are added to the 2 switches. These ports are configured with dynamic Vlans, that is the vlan id is set according to the MAC address of the notebook.