Computer networks -  Lab –5  Switching and VLANs.

Section 1: Configuring the Catalyst 2900 Switch

Configuring the 2900 switch is similar to configuring a router.

- You need to start the hyperterminal – using the settings:

  - 9600bps

  - 8 bit  No parity  1 Stop bit

  - Connect your PC  using the console cable to the console port of the switch.

- Once connected you:

  - Type  enable  -- to get into priviledge mode.

- To see the current configurations enter:

  - Type  show running-config

- To start configuring the switch you:

  - Type  config t  to enter the configuration mode

- Give the switch a name:

  - Type  hostname SwitchA

- To give it an IP address:

  - Type  int  VLAN1

ip address 172.19.128.1 255.255.192.0

- Now verify the parameters you just entered:

  - Type  cntl Z  to exit the configuration mode.

  - Type show running-config   to see the detail configurations.

Section 2: Configuring VLANs – on Catalyst 2900 Switch

Introduction.

A switch consists of many Ethernet ports (normally 10 Mbps). In the 2900 switch, the ports are fast Ethernet ports (100 Mbps). There are 12 fast-ethernet ports.

f0/1  f0/2	 f0/3  f0/4   f0/5   f0/6    f0/7   f0/8  f0/9 f0/10   f0/11  f0/12

Each of the port can be configured as either access port or trunk port.

Note access port only carries traffic for one VLAN, whereas trunk carries traffic for multiple VLANs.

In addition, each port can be configured to be belonging to different VLAN.

e.g.  f0/1 can be assigned as VLAN 2, while f0/2 can be assigned as VLAN 3.

To configure each port into a VLAN, perform the following:

- To start configuring the switch ports you:

- Type  config t  to enter the configuration mode

- To config a port, you enter the interface-config mode:

- Type  int f0/1    configuring the interface fast-ethernet 0/1

  - Type switchport access VLAN 2  an access port with vlan=2

For linkage between switches you need a trunk port:

- To set a port as a trunk port: (e.g. port 0/12)

- Type  int  f0/12

  - Type switchport mode trunk   set the port as a trunk port

  - Type switchport trunk allowed VLAN 1-12  a trunk port allowing vlans 1 to 12

Now we have 2 switches each switch will be assigned the following vlans, with port 12 assigned as the trunk port. The port 12 of the switches are interconnected to form the trunk link.

| port | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vlan | 2 | 2 | 3 | 3 | 4 | 4 | 5 | 5 | 6 | 6 | 7 | all |

| port | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vlan | 2 | 2 | 3 | 3 | 4 | 4 | 5 | 5 | 6 | 6 | 7 | all |

- Now using a long Ethernet cable, plug your computer into one of the switch port and configure the switch port to the vlan as shown in the table above.

- For testing, record your own ip-address and ask your friends ip address on same switch. Try pinging each other.

- Are you successful?

Record the :

Your ip  		Your Vlan	Neighbour’ ip         Neighbor’s VLAN   ping- Y/N

- Try pinging people from a different switch.

Record the :

Your ip  		Your Vlan	Neighbour’ ip         Neighbor’s VLAN   ping- Y/N